SafeCharge: Interactive BMS Digital Twin

Sprint 2 Plan — Group 6

Digital Engineering Summer Semester Project · Otto-von-Guericke-University Magdeburg

Document date: 14 May 2026 · Sprint 2 due: 22 May 2026

# Executive Summary

Sprint 1 closed with a complete concept lock for SafeCharge — an interactive 3D digital twin of a lithium-ion battery pack and its BMS, built in Unity with 3D assets authored in Blender. This document plans Sprint 2 (deadline 22 May 2026), which converts the concept into a first runnable scene: full process equations on the CEE side, initial 3D assets and a parameter-to-UI mapping on the DE/DKE side, and — based on the professor's Sprint 1 feedback — a new State-of-Health (SOH) module that exposes the gap between displayed SOC and the actual energy the pack can deliver, expressed as remaining EV driving range.

# 1. Sprint 1 Recap

Sprint 1 produced the concept presentation, the schematic process flow (Operator Input → Process Calculation → BMS Logic → Visualization → Result Card), the BMS architecture block diagram, the safety thresholds, and the first set of process equations (SOC integration and Joule heating). The CEE/DE task split was agreed: CEE owns the process math, the DE/DKE group owns the Unity build, Blender assets, and UI.

Status of Sprint 1 deliverables: concept, initial variables, simplified safety thresholds, Unity + GitHub setup, UI wireframes, and the flow-visualisation concept are all complete.

# 2. Professor's Feedback — Cell-Health Integration

The professor's Sprint 1 review pointed out that the current concept treats State-of-Charge as a single, honest number. In reality, the displayed percentage understates how degraded the pack actually is: when one or more cells have a lower State-of-Health (SOH), the pack's usable capacity is limited by the weakest cell in each series string. A pack reading 80% SOC may only deliver the energy that a healthy pack would deliver at, say, 64%.

Professor's recommendation — what the app must show Per-cell SOH visible in the cell-level view, not just pack-level SOC. Difference between displayed SOC (what the dashboard would normally show) and actual usable SOC under current SOH. A real-world consequence of that gap — e.g. an EV driving-range estimate that drops as cells degrade. Implication: the BMS scene must surface the asymmetry between cells. The user should be able to see which cells are pulling the pack down, and what that costs in km of range.

Section 4.3 and 4.4 below specify the math we will adopt; section 5.2 specifies the UI additions; section 6 lays out the Blender/Unity work needed to render it.

# 3. Sprint 2 Objectives

By 22 May 2026 the team will have:

- Full electrothermal equations documented and parameterised (SOC, Joule heating, simple lumped thermal update).

- SOH and weakest-cell-limits-pack math integrated into the process model.

- EV driving-range estimator wired to the SOH-corrected usable capacity.

- Initial Blender 3D assets for the battery pack and a representative prismatic cell, exported as .fbx for Unity.

- Unity scene scaffold loading those assets, with placeholder shaders for thermal colouring and a 16-cell SOH heatmap.

- UI wireframe updated to include: dual SOC gauge (displayed vs usable), cell-grid SOH heatmap, and range readout in km.

- Parameter-to-UI mapping table that each script and slider can be cross-checked against.

# 4. Technical Deliverables — CEE Side

## 4.1 State of Charge (SOC)

Coulomb-counting formulation, with sign convention: discharge current is positive.

SOC update (continuous) SOC(t) = SOC₀ − (1 / Q_nom) · ∫₀ᵗ I(τ) dτ Q_nom — nominal capacity (Ah); I(τ) — pack current (A); SOC in [0, 1]

Discrete-time form used by the Unity update loop SOC[k+1] = SOC[k] − (I[k] · Δt) / (3600 · Q_nom) Δt in seconds; the 3600 converts to Ah

## 4.2 Joule Heating and Lumped Thermal Update

Internal heat generation is dominated by ohmic loss in the cell internal resistance. The pack is modelled as a single lumped mass for Sprint 2 — per-cell thermal coupling is deferred to Sprint 4.

Heat generation rate (per cell) Q̇_gen = I² · R_int(SOC, T) R_int from a lookup table to be filled in Sprint 2; default 30 mΩ until populated

Lumped thermal balance m · c_p · dT/dt = Q̇_gen − h · A · (T − T_amb) m — pack mass; c_p = 800 J/(kg·K); h — convective coefficient (~10 W/(m²·K) passive, ~40 active cooling)

Thresholds (locked from Sprint 1): cooling fan activates at T > 45 °C; runaway alarm at T > 60 °C.

## 4.3 NEW — State of Health and Weakest-Cell Math

Each cell j carries an SOH ∈ [0, 1] representing its present full capacity relative to nominal. For lithium-ion cells, EOL is conventionally taken at SOH = 0.80.

Per-cell SOH SOH_j = Q_full,j / Q_nom Q_full,j — current full capacity of cell j after ageing

For cells in series (the topology we visualise), pack capacity is limited by the weakest cell — once that cell empties, the BMS must cut current regardless of what the others still hold. For parallel groups, capacities sum.

Usable pack capacity (series string of N cells) Q_pack_usable = minⱼ ( Q_full,j ) , j = 1 … N i.e. the worst cell sets the ceiling for the whole pack

Pack SOH (reported to the dashboard) SOH_pack = Q_pack_usable / Q_nom this is the figure compared against the 80% EOL line

The user-facing distinction the professor asked for is the gap between displayed SOC (what a naive gauge would show, computed from coulomb counting against Q_nom) and actual usable SOC (what fraction of the energy actually accessible from the pack remains).

Displayed vs Actual usable SOC SOC_actual = SOC_displayed · SOH_pack Example: SOC_displayed = 80 %, SOH_pack = 0.80 → SOC_actual = 64 %

## 4.4 NEW — EV Driving Range Estimator

To translate the SOC/SOH gap into something the user can feel, the dashboard will show remaining range in km. We pick a reference EV consumption figure (configurable in the scene) and apply the SOH-corrected usable energy.

Usable energy in the pack E_usable (kWh) = V_nom · Q_pack_usable · SOC_displayed / 1000 V_nom — nominal pack voltage; default 350 V for the demo pack

Estimated remaining range Range (km) = E_usable / c_drive c_drive — vehicle consumption in kWh/km; default 0.18 kWh/km (~mid-size EV)

Worked example (used as the demo scenario for the result card):

| Scenario | Value |
| --- | --- |
| Nominal pack | Q_nom = 100 Ah, V_nom = 350 V, SOH = 1.00 |
| Displayed SOC | 80 % |
| If pack is healthy | E = 350 · 100 · 0.80 / 1000 = 28.0 kWh → Range ≈ 156 km |
| If weakest cell SOH = 0.72 | Q_pack_usable = 72 Ah → E = 20.2 kWh → Range ≈ 112 km |
| Loss the user sees | ‑44 km on the same 80 % reading |

Note: The result card at the end of a scenario will display the two range numbers side by side so the user sees the cost of cell degradation directly.

# 5. Visualization & UI Deliverables — DE/DKE Side

## 5.1 3D Models (Blender)

- Battery pack outer casing (prismatic module shell, ~355 × 151 × 108 mm), low-poly with a separate top cover for the cell-grid reveal animation.

- Representative prismatic cell (×16) arranged in a 4×4 grid inside the module. Each cell is a separate object so per-cell shaders can drive SOH colour individually.

- Cell cross-section asset: anode / separator / cathode strips, ready to receive an ion-flow particle system in Sprint 4.

- Cooling fan unit (housing + 5-blade rotor, separated for spin animation).

- Export contract: .fbx with applied transforms, Z-up, 1 unit = 1 m, single material slot per object, named pack_shell, cell_##, fan_housing, fan_rotor.

## 5.2 UI Mockups — Updated for SOH Module

The Sprint 1 dashboard had a single SOC ring, a temperature card, current readout, and runaway-risk banner. Sprint 2 adds three elements driven by section 4.3 and 4.4:

- Dual SOC gauge — outer ring shows displayed SOC, inner ring shows SOC_actual. The wedge between them is the SOH penalty.

- Cell-grid SOH heatmap — 4×4 tile widget mirroring the 3D cell layout, coloured green (SOH ≥ 0.90) → yellow (0.80–0.90) → red (< 0.80). Hovering a tile highlights the matching 3D cell.

- Range readout — two numbers: range if pack were healthy, range with current SOH. The delta is the headline number.

## 5.3 Parameter-to-UI Mapping

This is the contract between CEE process math and DE Unity bindings. Every entry must be implemented by a script in Sprint 2.

| Symbol | Unit | UI element | Source |
| --- | --- | --- | --- |
| I | A (C-rate) | Current slider | User input |
| T_amb | °C | Ambient slider | User input |
| BMS mode | — | Mode dropdown | User input |
| SOC_displayed | % | Outer SOC ring | Eq. 4.1 |
| SOC_actual | % | Inner SOC ring | Eq. 4.3 |
| T_pack | °C | Temperature card + 3D shell colour | Eq. 4.2 |
| SOH_j (j=1..16) | — | Cell heatmap tile + 3D cell colour | Eq. 4.3 |
| SOH_pack | — | SOH bar in dashboard footer | Eq. 4.3 |
| Range_healthy | km | Range readout (grey) | Eq. 4.4 |
| Range_actual | km | Range readout (maroon) | Eq. 4.4 |
| Warning state | enum | Banner colour + text | BMS logic |

# 6. Blender & Unity Work Breakdown

Two of the DE/DKE members will own Blender authoring, two will own Unity integration. To make handover predictable we will commit assets and scripts via the GitHub repo set up in Sprint 1.

## 6.1 Blender — scripted asset generation

To keep the geometry reproducible (and to let CEE re-export if dimensions change), the battery pack will be built by a parameterised Blender Python script rather than by hand. The script lives at /assets/blender/build_pack.py. Inputs: cell rows, cell cols, cell width/height/depth, shell thickness, fan presence. Outputs: a Collection per logical object, ready for .fbx export.

- Run with: blender --background --python build_pack.py -- --rows 4 --cols 4 --out pack.fbx

- Each cell is named cell_00 … cell_15 to match the SOH array index used in Unity.

- Materials are placeholders — final PBR textures land in Sprint 4.

## 6.2 Unity — scene scaffold and scripts

Unity 2022 LTS, URP. Sprint 2 produces a single demo scene SafeCharge_Demo.unity containing:

- BatteryPackController.cs — owns the SOC, SOH and thermal state; runs the equations from §4 in FixedUpdate.

- CellGrid.cs — holds the 16-cell array, applies SOH colour to each 3D cell and to its heatmap tile.

- RangeEstimator.cs — pure function block, listens to BatteryPackController events and writes Range_healthy / Range_actual to the dashboard.

- BMSLogic.cs — threshold checks for 45 °C cooling activation and 60 °C runaway alarm; raises UI events.

- DashboardUI.cs — Unity UI Toolkit binding; subscribes to the events above.

Note: All scripts will be authored against an interface so Sprint 4 can swap the lumped thermal model for a per-cell coupled one without breaking the UI.

# 7. Task Assignment — Sprint 2

| Owner | Discipline | Sprint 2 task | Internal due |
| --- | --- | --- | --- |
| Rishi Chalamalasetty | CEE | Section 4.1–4.2 equations finalised; R_int lookup populated; default pack/V_nom values agreed. | 16 May |
| Avinash Lobo | CEE | Section 4.3–4.4 SOH math + EV range estimator; produce the worked-example numbers for the result card. | 16 May |
| Jashkumar Dhameliya | DE | Blender script (build_pack.py): pack shell + 4×4 cell grid + fan; .fbx export verified inside Unity. | 18 May |
| Shashi Kumar | DE | Unity scaffold: scene, BatteryPackController, BMSLogic, FixedUpdate loop wired to §4 equations. | 20 May |
| Anant Sharma | DKE | CellGrid + RangeEstimator scripts; SOH heatmap UI widget; parameter-to-UI mapping table validated in-engine. | 20 May |
| Hari Charan | DE | DashboardUI: dual SOC gauge, range readout, warning banners; integration test with sample SOH array. | 21 May |

Note: Internal due dates leave 22 May for joint dry-run and slide preparation.

# 8. Risks & Dependencies

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Blender .fbx import breaks naming/scale in Unity | Medium | High — blocks all visualisation work | Scripted export with applied transforms; smoke-test import on day one. |
| R_int lookup data unavailable in time | Medium | Medium — thermal curve will be unrealistic | Use a constant 30 mΩ default; flag values as 'placeholder' in the UI. |
| SOH demo numbers feel arbitrary to the professor | Low | Medium — undermines the new feature | Anchor the example to a real cited EV (consumption, pack voltage) and show source in result card. |
| Unity URP shader compile errors across machines | Medium | Medium — slows iteration | Commit Library/PackageCache exclusions; pin URP version in manifest.json. |
| Team-member calendar conflict before 22 May | Low | Low — small slippage | Internal dues 1–2 days before sprint deadline build slack into the schedule. |

# 9. Looking Ahead — What Rolls into Sprint 4

Sprint 4 (deadline 19 June 2026) will take the Sprint 2 scaffold to a validated, animated demo:

- Replace lumped thermal model with per-cell coupled electro-thermal, so the SOH heatmap and the temperature shading drive each other.

- Ion-flow particle system on the cell cross-section, speed mapped to I (as in the Sprint 1 mockup).

- Result card UI with the protected vs unprotected scenario comparison.

- PBR materials on Blender assets; cooling fan rotor animation tied to the 45 °C threshold.

- Final 3D model import (frozen).

Definition of done for Sprint 2: a runnable Unity scene where moving the current slider changes SOC, T_pack, and the EV range readout, and where editing the SOH array produces a visible difference between displayed and actual range. The professor's feedback is then visibly addressed by the headline number on screen.